Stebel Horn fitment

This method was used by me when I replaced the OEM horn with the much louder 139dB Stebel horn on my 2000 Suzuki Intruder LC.

- First removed the left faux tank.

- Remove the OEM horn.

- Mount the stebel horn in its place on the stock bracket. Note that the horn must be fitted with the air cylinder vertically to reduce wear on internal parts.

- Move the original horn wiring back up towards the frame. These two wires are one hot (+) lead and the other the ground (trigger) which comes from the horn button on the handles.

- Connect the hot lead the relay number 86.

- Connect the trigger lead to number 85.

- Connect an additional 14 Ga wire from the starter relay positive to a 20 Amp inline fuse and onto number 30 on the relay.

- Connect an additional 14 Ga wire between 87 of the relay and the (+) of the Stebel horn. Here a red colored wire can be used.

- Connect an additional 14 Ga wire to ground and to the (-) side of the Stebel horn. Here a black colored wire can be used.

- The relay can now be tucked nicely away in the space just behind the fuel pump area.

- At this point you should test the horn for correct operation.

- The left side faux tank can now be replaced.

- Used a black cable tie and neatly applied this to the horn OEM bracket with the wiring behind the bracket.

Looks neat and sounds awesome!

By Tiny™ (Leon Olivier)