C90 RCF INSTALL

Okay, before we start, this installation requires drilling and tapping 4 holes in the triple tree.  Also, you will need to trim some areas around the headlight area of the RCF.  I am not responsible for errors that might be in this description or for any misinterpretation of these instructions.  If you are still going ahead with this, then here we go.

1.  The mounting kit comes with metric mounting bolts.  If you are going to use these bolts, you will have to get a metric tap and die kit.  The kit will tell you what size hole to drill in order for it to be taped.   If you wanted to use standard, you would have to determine size on your own.

2.  We will start on the right side of the bike.  If you look at the lower triple tree on the back side you will see where the tree clamps on the tube.  Just to the left of that is a flat area with what looks like a half circle on the bottom.  It is the area circled in this picture.  On your bike there will not be a hole located here.  Imagine that that half circle were completed in to a full circle.  The center point of this imaginary circle is the center point for our mounting hole.

The bolts for the bottom mounts are 6mm-1.0.  You will have to refer to your tap and die kit for the correct size hole to drill.  Drill and tap this hole and then repeat on the left side of the triple tree.  When drilling, you only need to drill deep enough to get to the hollow area.  Don’t continue to drill or you will come out the front of the triple tree.

3.  Once these holes have been drilled and taped, you should attach your mounting brackets and tighten loosely so you are able to move them around with a little resistance.  Rotate the upper part of the brackets in front of the upper triple tree.  (You may need to move your cables in order to do this.)  When this is done it should look like this.

At this point take your fairing and put the two mounting bolts of the fairing into the holes on the top of the mounting bracket.  We are not concerned with the headlight portion of the fairing at this point.  Attach the nuts to the fairing where you just set it on the mounting bracket.  At this point just move the fairing back and fourth until it is centered.  At that position pull the mounting brackets up tight to the front of the triple tree and secure them in place, either with tape or a clamp.  Be careful if you use a clamp to not scratch your upper triple tree.   Make sure that the fairing is still centered and that the mount securing points for the mounting brackets look to be equal distance from the end of the triple tree on both sides.  Again refer to the picture above. Make sure the mounting brackets are secure and now remove just the fairing.  You now know where to drill the holes in the upper triple tree.

4.  The upper mounting bolts are 8mm-1.25.  After determining what size hole to drill, just use the mounting brackets as guides and drill your holes.  Make sure you use the brackets holes as guides so that your holes are in the right position and at the proper angle to the triple tree.  Again, only drill deep enough to get to the hollow area inside the triple tree.  After these holes are drilled, remove or rotate the brackets to the side and then tap the holes to the appropriate size.

5.  Okay, now put the brackets back into position and secure them loosely, but with some tension.  Place the fairing in place and attach loosely.  If everything worked out the fairing should be centered on the triple tree but the front of the fairing will be sitting up on the headlight.   You will now need to trim the front of the fairing to get it to fit around the headlight.  Trim a little at a time until you can get the lower part of the fairing ears to contact the mounting bracket.  There are little ears on the lower part of the fairing that have to be drilled to attach to the mounting brackets.  Push the faring down until these ears are in position and then drill up from the bottom through the holes in the mounting bracket through the fairing ears.  Do this on both sides of the fairing.  Place your bolt from bottom up through the bracket and fairing and secure with a washer and nut.  You should now be able to tighten everything up and have RCF installed on your C90.

If you have any questions please feel free to contact me.  My email is wrclark@acmenet.net.  When you send the email you will get a request from me in your email to verify your not just junk mail.  Also, in your subject put “C90 RCF Install”

Thanks